# Software VTS. Navi-Harbour

The solution proposed herein is based on Wartsila Navi-Harbour SW which is designed for Vessel Traffic Management Systems (VTMS). Navi-Harbour has numerous installations in world’s busiest ports, thus it is designed to fulfil operational requirements in ports with sophisticated navigational situation.

## Operating Principles

The operating principle consists in:

reception of information on the navigational situation from the external sources referred to as SENSORS;

generalizing of the target data;

presentation of the target data in the Target Table;

presentation of graphic target data in combination with the multi-layer electronic charts of the area;

analysis of the navigational situation and generation of alarms and warnings according to the criteria set by the operator;

output of various auxiliary navigational and other information;

digital recording of the navigational situation data for the further playback.

For security purposes, the accessibility of the system will be developed by following hierarchy architecture, segregated by the use of passwords (In precedence order):

VTMS Operation level (Disabled: Chart editing; Access level editing; Parameters of VTS sensors connection; Navigational zones adding, changing or deleting; Default alarms settings; Logging control)

VTMS Supervisor level

VTMS Management Level

Any number of levels with any restrictions can be created

## System Interface

### Operators and Security lists.

Navi-Harbour uses local data base to store list of users and security levels. Security levels are not associated with specific operators. Operators and security levels are protected with passwords.  Any operator can change own security level to perform some advanced actions. It is recommended to use minimal security level for daily activities.

List of default permissions is shown in the table:

| Operation | Administrator | System engineer | Operator |
| --- | --- | --- | --- |
| VTS chart editing | Yes | Yes | – |
| Sensor chart editing | Yes | Yes | – |
| Nautical chart editing | Yes | Yes | – |
| User chart editing | Yes | Yes | – |
| ODU configuration | Yes | Yes | – |
| Sensor configuration | Yes | Yes | – |
| Network configuration | Yes | Yes | – |
| Default alarm setting | Yes | Yes | – |
| List of operators | Yes | Yes | – |
| VTS operation modes | Yes | Yes | – |
| System security parameters | Yes | – | – |
| Logging control | Yes | – | – |
| Closing of VTS Server program | Yes | Yes | – |
| Closing of VTS Client program | Yes | Yes | Yes |
| Sensor control. Radar control – general | Yes | Yes | Yes |
| Sensor control. Radar control – advanced | Yes | Yes | – |
| Sensor control. CCTV control – general | Yes | Yes | Yes |
| Sensor control. CCTV control – advanced | Yes | Yes | – |
| Sensor control. RDF control – general | Yes | Yes | Yes |
| Sensor control. RDF control – advanced | Yes | Yes | – |
| Changing of chart settings | Yes | Yes | – |
| Changing of message options | Yes | Yes | – |
| VTS Server selection | Yes | Yes | – |
| Setup system status messages | Yes | – | – |
| Worksapace control – general | Yes | Yes | Yes |
| Worksapace control – advanced | Yes | Yes | – |
| Worksapace control – save as default | Yes | – | – |
| Target symbol presets editing | Yes | Yes | – |
| Changing of target classes configuration | Yes | Yes | – |
| Target identification | Yes | Yes | Yes |
| Target classification | Yes | Yes | Yes |
| Target dropping | Yes | Yes | Yes |
| Database operations | Yes | Yes | Yes |

It is possible to access any settings of ODU 0 from any ODU N.

### General Description of System Interface

Graphical interface of the system is based on Windows technology.

There are following basic elements of the interface:

Entire operator activities are accessible via system menu. All information tables also have context menus. All menus can be customized by system operator.

Most useful operations are accessible with single click buttons grouped in toolbars corresponding to its application. Buttons on toolbars can be customized by operator. Following toolbars are available. Following toolbars are available in the default configuration:

Chart operations.

Base chart editor.

Chart editor.

Extended chart editor operations.

Target information.

Target tracking.

Target prediction/simulation.

Figure 6-1 Target prediction

Logging.

Database operations.

CCTV cameras control.

Multiple Chart Windows can be displayed on Operator Display Unit (ODU) monitors with possibility to select size, position and displayed area for each of them.

Specific data is distributed in a set of information tables:

Tracked target table.

Buoy targets table.

Fixed targets table.

Navigational alarms table.

Hardware alarms table.

Target pair table.

Aids-to-Navigation table.

AIS target table.

Target search table.

Special data panels are displaying numeric navigational information:

Cursor data panel.

Date and Time panel.

Reference target data panel.

Route data panel.

CPA\TCPA data panel.

System diagnostics panel is displaying on-line diagnostics for system main components and connected sensors.

Hot-key combinations are assigned to most operations.

ODU execute also the following functions:

Presentation of digitalised geographical map/chart as underlay in the main display, and overlapping the combined radar video / track on it;

Presentation of target tracks from Radar as well as UAIS;

Generation of complete situational awareness picture of the area of interest, to enable surveillance and monitoring;

Generation of automatic alarm when targets cross pre-set alarm criteria or a predefined surveillance area set on the display;

Remote control of individual radar parameters;

Database management of targets/tracks;

‘Multi sensor data fusion’ allowing correlation of data from both the radars as well as UAIS.

Figure 6-2 Navi-Harbour Main Window

Figure 6-3 Navi-Harbour Chart Layers

Figure 6-4 Navi-Harbour Chart Window

Dedicated VTS operator keyboard provides one touch access to major operator activities.

### Chart Window

General Description

Operator can create one or several (up to 8) independent Chart Windows. Each Chart Window displays following set of information in VTS area:

Navigational charts.

System charts.

Tracked targets.

Radar video overlay.

Other visual information.

Operator can save, close, rename, resize any Chart Window or move it to particular place on the desktop.

Supported Navigational Charts

The following navigational chart formats are supported:

Vector TX-97 chart format. TX-97 vector data (TX-97 format charts) are produced according to Wartsila standard. The production of TX-97 data is organized according to ISO 9001 product quality management standard.

S-57 electronic charts (IHO S-52 standard presentation) converted to internal .cd8 format using Chart Assistant utility.

GeoTIFF files (i.e. satellite images).

ERSI Shapefile (i.e. data from Geodetic Informational Systems).

All supported charts are storing in VTS-server and distributing to each ODU of the system.

Electronic chart includes following objects:

Coastline.

Land objects.

Depth contours.

Depths.

Buoys.

Lights.

Warning points.

Caution areas.

Anchorage areas.

Traffic separation schemes etc.

Target Data Presentation

System provides continuous monitoring of all tracked targets' positions, vectors and displays this data on the electronic chart. Operator can choose following options for targets displaying:

Selectable target symbol (dot\scalable\bitmap).

Special target symbol for target tracked by AIS, radar or AIS+radar data.

Target text data (ID, course, speed, heading, AIS report rate) with adjustable position relatively to target symbol.

Target vector with adjustable length.

Target history with selectable length, type of history line and time marks.

Graphical AIS data presentation (heading, vessel dimensions, GNSS antenna position, rate of turn).

Vessel outline based on real dimensions (if available).

Possibility to assign different target symbols to particular vessel types.

Vessel safety domain.

Figure 6-5 Targets

Figure 6-6 Legend

Figure 6-7 Target Direction

Radar Video Picture Presentation Over Electronic Charts

System provides to operator presentation of multi-radar video picture over electronic charts. Operator can choose following options to display this picture:

Possibility to select the radars for simultaneous presentation of video picture or mosaic radar picture.

Possibility to select transparent radar video presentation with selectable density level (25% - 100%).

Radar video afterglow presentation with color fading. Adjusted afterglow length (from 0 to 400 scans or unlimited).

Selectable scan-scan correlation for video presentation.

Highlighting of persistent target functionality to improve visibility of echoes from small targets.

Multi-radar video picture presentation can be displayed in overlapping or mosaic mode.

Fill gaps mode (sweep-sweep approximation).

Suppression of shore radar echo (video masking zones).

Possibility to change default radar video presentation colors. This functionality is available on request.

Figure 6-8 Target Display Options

Figure 6-9 Radar Video Presentation

### Information Tables

Informational tables contain text data about particular types of system objects. All tables can be adjusted by size, number and width of columns. Operator can choose column to sort data in preferred order. Tables can be switched to “Always on top” mode above of the Chart Windows. Each table has own context menu and provide fast access to most operations. Tables can be printed.

Target Table

Target table contains data about all targets acquired by system (manually by operator or automatically). Operator can locate target in a table using special search panel. Target table has additional optional “Selected target info” panel.

Figure 6-10 Target Table

AIS Target Table

AIS target table contains all AIS targets in a system coverage area (including SAR aircrafts) even if they aren’t acquired. Operator can locate target in a table using special search panel.

Figure 6-11 AIS Target Table

Fixed Target Table

Fixed targets table contains information about radar fixed targets. This data can be used for radar calibration procedures

Figure 6-12 Fixed Target Table

Aids-to-Navigation table

Aids-to-Navigation table contains information about A-to-N object equipped by AIS in a coverage area.

Figure 6-13 Aids-to-Navigation Table

Navigational alarms table

Navigational alarms table stores list of navigational alarms.

Figure 6-14 Navigational alarm Table

Hardware alarms table

Hardware alarms table stores list of sensor alarms are below:

Figure 6-15 Hardware alarms Table

Target Pair Table

Targets with CPA/TCPA alarms are automatically adding to Targets pair table. Also, operator can manually add any number of targets pairs for continuous monitoring their relative position. Targets can be easily removed from this table if necessary.

Figure 6-16 Targets pair table

Target Search Table

Operator can determine one or several criteria. Targets comply with these criteria will be added to this table. Operator can switch on continuous highlight mode for these targets.

### Data panels

Data panels provide to operator set of numeric data. There are following data panels available:

Cursor data: continuously display of geographic coordinates and relative position of cursor in a chart window.

Reference target: can be opened for selected target. This panel displays coordinates, course and speed of this target. For each Chart Window operator can open one panel.

CPA/TCPA: can be opened for selected target, if another target was chosen as reference. This panel displays CPA/TCPA for selected pair of targets.

Route: can be opened for selected routed target. This panel displays DTE, XTE and other route parameters.

### System diagnostic

System provides to operator diagnostic of main components and sensors. Short diagnostic is located in a status bar.

Advanced diagnostic can be checked in a special diagnostic panel. All critical events in this panel appear as alarms in hardware alarms table.

Figure 6-17 System Diagnostic Panel

## Basic chart operations

### System diagnostic

The following chart operations are available in a Chart Window:

Any set of licensed charts can be displayed for each Chart Window independently (loading/unloading through the operator toolkit).

User-defined set of chart overlays is adjustable for each Chart Window independently.

Smooth scaling, arbitrary off-centering and electronic chart alignment (60 selectable scales from 1:1000 to 1:1000000000).

Rotation of the area’s chart display.

Multi-step undo of previous changes of scale, off-centering etc.

Fast zoom to any selectable area inside of Chart Window.

Instant Zoom facility with adjustable zoom ratio and size of zoom area.

Constant Zoom of some particular area or vessel (with “follow” function) in a separate adjustable window.

General View window for fast navigation inside of responsibility area.

Operator can choose between 4 different palettes (Day, Twilight, Dusk, and Night), depend on work conditions. These palettes include colors of objects in a Chart Windows, targets, radar video, text in tables and background.

Figure 6-18 Color Patterns

Wide variety of screen focusing on desired area (focusing on the target, buoy, group of targets etc.).

System allows operator to choose between unlimited numbers of predefined reference points or select any geographic position as a reference point using corresponding tool.

Displaying of coordinates grid.

Displaying of range rings centered on operator defined point (reference point).

Electronic range and bearing line (ERBL). Operator can choose different ERBL modes: floating and fixed tooltip, different measurements unit, scalable measurements units (unit depends on measurement distance).

Fast switching between observe areas using “predefined area” tool.

Displaying information about any chart object.

Printing any Chart Window or information table with several print modes.

Selecting of measurement units incl. imperial units (NM, cables, km, m, feet, yards, knots, km/h, m/s etc).

Selecting date/time format.

## Target operations

### Basic target operations

The following set of tools allows working with targets:

Automatic target acquisition (any set of predefined polygonal auto-acquisition zones).

Manual target acquisition.

Target identification (entering static vessel data) manually or attaching to the VTS DB entry.

Displaying full set of target’s information in a special panel.

Automatic target identification for AIS targets.

Target auto drop facility.

Manual target drop.

“Follow” tool for target of special interest.

Reference target tool for continuous monitoring of target’s dynamic data.

CPA\TCPA on-line calculation for any targets pair.

ETA table for all targets approaching selected arbitrary point.

ERBL tool attached to moving target with course angle displaying.

Target identification data exchange tool for swapped targets.

Target pair merging tool for lost target reacquiring.

Destination point tool: possibility to select for every target the arbitrary destination point and continuously monitor the approaching parameters (distance, course deviation, ETA etc.).

“Dangerous cargo” status can be assigned to the target and allows operator to highlight targets with dangerous cargo or hazard materials on board.

“Service ship” status can be assigned to the target and allows operator to avoid alarms from port tugs, pilot boats and other service vessels in VTS area.

### Target tracking modes

There are four target tracking modes: free, routed, anchored and moored.

Free mode: is a standard tracking mode, assigned to all acquired targets.

Figure 6-19 Free mode

Anchored mode: is assigning to target at anchor. In this mode system will monitor position of target and anchor domain around it with generation of anchored alarms. This mode can be assigned manually by operator or automatically from AIS status.

Figure 6-20 Anchored mode

Routed mode: is assigning to target moving on a predefined route.

Figure 6-21 Routed mode

Traffic routes can be created in the system with following parameters:

Allowed traffic directions.

Speed limits on route.

Safety domain radius for route vessels.

Channel edges and depth.

Allowed deviation (XTE and DTE).

Autoroute parameters.

Target can be assigned to route manually (by operator) and automatically (autoroute function). System provide to operator following functions for routed targets:

Calculation of XTE and DTE for all routed targets with generation of alarm if XTE or DTE is dangerous for target safety passage.

Monitor target course and speed with generation of alarm in case of parameters violation.

Display target ETA schedule for all route reference points.

Display route reference point ETA schedule for all passing targets.

Operator can use Route Profile – sophisticated route planning tool. It provides following advantages:

Graphical display of all targets along the route in range/time co-ordinates.

Calculation and easy modeling of targets’ related movement.

Specific route segments support (meeting prohibited, route reference point, etc.)

Moored mode: is assigning to moored target. This mode allows operator to plan mooring operations. Target can be moored to predefined berths. This mode can be assigned manually by operator or automatically from AIS status (in this case system will choose closest berth to target’s position).

Figure 6-22 Moored mode

### AIS data handling

When the system equipped with one or several AIS data source the following ODU functionality is provided:

Displaying of Class A and Class B AIS targets.

Auto identification of the target with AIS data.

AIS + radar target data integration with possibility to choose type of data used for tracking (AIS, radar or AIS+radar).

Displaying static vessel information received from AIS source.

Interrogation (manual), receiving, processing and displaying of AIS extended static information sent using Binary Messages according to IMO Circular SN236:

Number of persons onboard.

Dangerous cargo information.

Air draught.

Auto identification of AIS data with VTS database record.

Sending\Receiving of safety-related and text AIS messages.

AIS target report rate control.

Automatic changing of tracking mode (free, anchored) from AIS data.

Automatic changing Dangerous Cargo status from AIS data.

Displaying of Search and Rescue Aircrafts with special symbol.

Displaying of Aids-to-Navigation objects with special symbol.

### AIS services

Man-Over-Board (MOB) over AIS service

This feature allows reception of MOB bearings from Wartsila Navi-Sailor over binary AIS messages. Several ships equipped by RDF sensors and AIS-transponders receive signal from MOB beacon and transfer it to coastal station. Operator can locate estimated position of the beacon if at least 2 bearings are available. System generates alarm if beacon appeared. This complex solution allows locating overboard person and organizing SAR operation.

### Prediction and simulation

Additional set of target management tools allows operator several advanced functions:

Target position prediction provides calculation of all targets positions according to the current target’s mode (en-route calculation for the targets assigned to the route) for unlimited time ahead using the mouse.

Trial Maneuver tool allows operator to simulate changing speed or course of two targets to avoid collision between them.

Figure 6-23 Trial Maneuver tool

Destination point allows operator continuously view course angle and ETA from selected target to any geographic point.

Figure 6-24 Destination point

Operator can create simulated targets by setting of position, course and speed.

For every created simulated target operator can change position, course and speed or drop it.

Figure 6-25 Changes of simulated target

### Navigational alarms

Figure 6-26 Navigational alarms

System has set of navigational alarms intended to avoid accidents in coverage area.

All alarms appear in the Navigational alarms table and attended by audible alert.

Alarmed targets also indicated in a Chart Windows and target table. Operator should acknowledge alarm to switch off sound alert. For each particular type of alarm is possible to change alarm criteria, alarm sound and priority. There are 4 priorities supporting to operator in decision of the alarm importance. Alarm sound can be selected from 3 predefined sounds. Operator can load additional sound files in a standard .wav format and use them in alarms generation.

Operator has the possibility to attach individual alarm setting to any target or group of targets.

To avoid unnecessary alarms from port tugs, pilot boats and other service vessels, operator can assign to corresponding targets the “Service ship” status.

The following list of alarms is available in a system.

Standard alarms:

CPA/TCPA

Safety domain breach, free mode collision danger

Lost on sensor

Target acquired

Target dropped

Alarms for routed targets:

XTE out on route

Dangerous DTE

Collision danger, meeting on route

Collision danger, meeting on turn point

Collision danger, overtaking on route

Grounding on route

Course out of route

Speed limit breach on route

Enter/leave route line

Approaching to turn point

Alarms for anchored targets:

Anchor dragging

Anchor domain violation

Alarms for AIS targets:

Low position accuracy

Navigational status changed

Tracking mode has been changed

Incoming AIS message (safety related)

Incoming AIS message (binary)

Alarms for buoys, A-to-Ns and fixed targets:

Buoy off-position

Buoy lost

Fixed target dangerous drift

Fixed target off-position

A-to-N off-position

A-to-N low position accuracy

Traffic zone alarms (number of zones does not limited):

Speed limit

Target decrease speed

Target increase speed

Suspect anchoring

Restricted course

Transit time violation

Number of targets in zone exceed limit

Sudden turn

Guard zone alarms (number of zones does not limit):

Approach to zone border outside

Approach to zone border inside

Entering zone

Leaving zone

Being in zone

Reporting line crossing

### Hardware alarms

System provides to operator diagnostic of main components and sensors. All critical events appear as alarms in hardware alarms table. Operator can mask any unwanted alarm.

### Automatic sending of AIS messages

System provides possibility to send automatically safety related AIS message to ship than it is crossing predefined reporting line.

## CCTV Sensor Operations

### Focusing on objects

Operator can focus on targets or any chart objects.

Figure 6-27 Focusing on target procedure and graphic presentation

### Automatic following

Operator can lock the camera on target.

The CCTV camera symbol and the observed object will be connected with a dashed orange coloured line, and the CCTV camera will be turning automatically until this line is coincident with the CCTV camera’s bearing line which is plotted on the chart as a solid orange line.

The camera’s viewing angle lines are shown in small dashes, and the camera’s visibility limits - in a dash-and-dot line.

Figure 6-28 Follow procedure

The CCTV camera will be turning automatically keeping the target inside the angle of view.

When the Following Auto mode or the icon is selected but there is no free CCTV camera in the responsibility zone at this moment, then following of the target will start when

Any camera is released in the responsibility zone

The target moves to another responsibility zone.

To cancel the follow mode, do either of the following:

1. Click the icon of the CCTV camera which followed the target.

2. Clear the check mark in the target context menu or in the table context menu.

The follow mode is cancelled automatically in the following cases:

In the CCTV Cameras control window, the pan or tilt angle has been changed

The target has been dropped or lost

Another ODU has taken over the CCTV camera control.

### Auxiliary

#### CCTV Camera Control Window

To open the CCTV Cameras control window:

ff Choose the Sensor > CCTV Cameras menu item. The CCTV Cameras - <camera name> window opens up.

The window title bar shows the name of the CCTV camera selected in the tree view in the left part of the window, and the name of the target tracking or focusing-on-target operation if any is currently performed by the CCTV camera.

The pairs of fields show the CCTV camera pan and tilt angles, zoom and focus:

The left hand field shows the current parameter value

The right hand field – a value, set by the operator, which is

displayed until the current value becomes equal to it.

CCTV camera the Master button is in the “depressed” state .

Figure 6-29 Camera control interface

#### Auxiliary tab

To control the CCTV camera focus use the buttons  or the   <Page Up> and <Page Down> keyboard keys.

With the button depressed,  the automatic focus selection mode is ON. The automatic focus selection mode is run by default at the program start. To control the focusing manually, release the  button: .

- to turn on the camera glass wipers.

- to turn on the camera glass washer.

In addition to these buttons, other control buttons can be set up for your camera depending on the camera configuration.

Figure 6-30 Axillary

## RDF Operations

Figure 6-31 Direction finder sensor visual representation

### Control tab

You can select channels you want to monitor on this tab.

Figure 6-32 Control tab

### Display tab

You can configure display settings of visual data representation, Line settings, etc.

Figure 6-33 Insert Tab

### Diagnostic for DF Sensor

Figure 6-34 DF Sensor Alerts

If the Generate an alarm checkbox is selected in the sensor properties for the direction finder, the “DF<Name>: CH <Name>- <Alarm text> on <Bearing value>” alarm is generated:

DF<Name> - direction finder name

CH<Name> - channel name

<Bearing value> - bearing angle value at the alarm generation moment.

The following alarms can be generated for the direction finder:

Table 1 DF Alarms

| ALARM TEXT | NOTES |
| --- | --- |
| DF Server Failure | DF Server Failure No connection with the DF Server |
| No data | No data arrives via the selected channel during the time interval depending on the direction finder type |
| Channel error | Error connected with the channel, depends on the direction finder type |
| DF error | Error not connected with the channel. For example, “no connection to controller” |

## Recording/ Playback Facility

### Recording system

System provides continuous synchronous recording of following data in the log files:

Radar video.

Other VTS sensor data (AIS, DF, Meteo etc).

Targets' tracks.

System generated notifications/alarms.

Events.

Operator’s activities: desktop configuration (chart

System configuration and status.

Multi-channel audio data from VHF communication subsystem.

Recording system automatically creates multi-volume archive on shared media. Maximum length of volume is limited only HDD size of the recording system.

Figure 6-35 Recording Settings

Figure 6-36 Playback panel

Figure 6-37 Recording Panel

### Playback system

Playback ODU is a software which can be integrated in Navi-Harbour ODU (can be running in parallel with On-line work without interference) or used as a stand-alone Debriefing System.

Figure 6-38 Playback ODU

Playback ODU provides presentation of the all recorded data (playback) in an ODU ordinary graphic environment (all usual operations such as radar image control, scaling, off-centring, electronic measurements are accessible in the playback). Following additional features are available during playback:

Selecting playback fragment.

Increasing/decreasing speed of playback.

Displaying of recorded tracks with possibility to select any combination of tracks for playing back.

Searching for recorded fragment by user specified criteria (target name, MMSI, Callsign).

Saving selected record fragment to specified drive\folder.

Export tracked target data from selected fragment to text format.

Export recorded audio data to Wav-format

Printing out of graphic image during playback.

Capture video-file from playback data.

Audio records analysis mode provides wide set of features for analysis of voice records:

graphic presentation of voice activities in time line;

continuous playing back of audio records without “holes”;

selecting of audio fragment for continuous repeating.